QUEEN’S UNIVERSITY BELFAST

Safety Service

April 2014

Guidance on the Globally Harmonised System of Classification and Labelling of Chemicals

- Introduction

The Globally Harmonised System (GHS) of classification and labelling of chemicals is a worldwide initiative to promote standard criteria for classifying chemicals according to their health, physical and environmental hazards and communicating those hazard elements in the form of labelling and safety data sheets.

It uses pictograms, hazard statements and signal words to communicate hazard information on product labels and safety data sheets in a logical and comprehensive way.

The primary goal of the GHS is better protection of human health and the environment by providing chemical users and handlers with enhanced and consistent information on chemical hazards.

- Scope

The GHS applies to pure chemical substances, their dilute solutions and to mixtures of chemical substances.

The GHS system is built on 16 physical, 10 health and 3 environmental hazard classes and comprises the following communication elements:

- Pictograms

- Signal Words

- Hazard Statements

- Precautionary Statements

- Safety Data Sheets

- Labels

- Pictograms

Pictograms convey health, physical and environmental hazard information, assigned to a GHS hazard class and category. Pictograms will have a black symbol on a white background with a red diamond frame. For transport, pictograms will have the background symbol and colours currently used in the UN Recommendations on the Transport of Dangerous Goods.

There are now only 9 pictograms, which can be viewed below with an approximate translation of old to new.

| Description | Old Pictogram | New Pictogram | Health, Physical and Environmental Hazards |  |
| --- | --- | --- | --- | --- |
| Explosive |  |  | - Unstable explosives - Explosives of Divisions 1.1, 1.2, 1.3, 1.4 - Self-reactive substances and mixtures, Types A, B - Organic peroxides, Types A, B |  |
| Flame |  |  | - Flammable gases, category 1 - Flammable aerosols, categories 1, 2 - Flammable liquids, categories 1, 2, 3 - Flammable solids, categories 1, 2 - Self-reactive substances and mixtures, Types B, C, D, E, F - Pyrophoric liquids, category 1 - Pyrophoric solids, category 1 - Self-heating substances and mixtures, categories 1, 2 - Substances and mixtures, which in contact with water, emit flammable gases, categories 1, 2, 3 - Organic peroxides, Types B, C, D ,E, F |  |
| Oxidising |  |  | - Oxidizing gases, category 1 - Oxidizing liquids, categories 1, 2, 3 |  |
| Compressed Gas |  |  | - Compressed gases - Liquefied gases - Refrigerated liquefied gases - Dissolved gases |  |
| Corrosion |  |  | - Corrosive to metals, category 1 - Skin corrosion, categories 1A, 1B, 1C - Serious eye damage, category 1 |  |
| Toxic |  |  | - Acute toxicity (oral, dermal, inhalation), categories 1, 2, 3 |  |
| Irritant |  |  | - Acute toxicity (oral, dermal, inhalation), category 4 - Skin irritation, category 2 - Eye irritation, category 2 - Skin sensitisation, category 1 - Specific Target Organ Toxicity – Single exposure, category 3 |  |
| Health Hazard |  |  | - Respiratory sensitization, category 1 - Germ cell mutagenicity, categories 1A,1B, 2 - Carcinogenicity, categories 1A,1B, 2 - Reproductive toxicity, categories 1A,1B, 2 - Specific Target Organ Toxicity – Single exposure, categories 1, 2 - Specific Target Organ Toxicity – Repeated exposure, categories 1, 2 - Aspiration Hazard, category 1 |  |
| Environment |  |  | - Acute hazard to the aquatic environment, category1 - Chronic hazard to the aquatic environment, categories 1, 2 |  |

- Signal Word

‘Danger’ or ‘Warning’ will be used to emphasis hazards and indicate the relative level of severity of the hazard, assigned to a GHS hazard class and category. Some lower level hazard categories do not use signal words.

Danger – more severe hazards

Warning – less severe hazards

- Hazard Statements

Hazard statements are standard phrases assigned to a hazard class and category that describes the nature of the hazard. Each hazard statement has a corresponding identification code which consists of one letter and three numbers as follows:

- the letter ‘H’ (for ‘Hazard Statement’)

- a number designating the type of hazard as follows:

- ‘2’ for physical hazards

- ‘3’ for health hazards

- ‘4’ for environmental hazards

- two numbers corresponding to the sequential numbering of hazards arising from the intrinsic properties of the substance or mixture, such as explosive properties (codes from 200 – 210), flammability (codes from 220 – 230) etc.

A full list of hazard statements can be found in Appendix 1.

- Precautionary Statements

Precautionary statements are measures to minimise or prevent adverse effects. There are four types of precautionary statements covering: prevention, response in cases of accidental spillage or exposure, storage, and disposal. The precautionary statements have been linked to each GHS hazard statement and type of hazard.

Precautionary statements have been assigned a unique alpha numerical code which consists of one letter and three numbers as follows:

- the letter ‘P’ (for ‘Precautionary statement’)

- one number designating the type of precautionary statement as follows:

  - ‘1’ for general precautionary statements

  - ‘2’ for prevention precautionary statements

  - ‘3’ for response precautionary statements

  - ‘4’ for storage precautionary statements

  - ‘5’ for disposal precautionary statements

- two numbers corresponding to the sequential numbering of precautionary statements.

A full list of precautionary statements can be found in Appendix 2.

The ‘Hazard’ and ‘Precautionary Statements’ replace all risk and safety phrases.

- Safety Data Sheets (SDS)

The safety data sheet (formally known as ‘Material Safety Data Sheet’) is specifically aimed at use in the workplace. It should provide comprehensive information about the chemical product that allows employers and workers to obtain concise, relevant and accurate information. The SDS should contain the following 16 sections in the order specified:

- Identification

- Hazard(s) identification

- Composition / information on ingredients

- First aid measures

- Fire-fighting measures

- Accidental release measures

- Handling and storage

- Exposure control / personal protection

- Physical and chemical properties

- Stability and reactivity

- Toxicological information

- Ecological information

- Disposal considerations

- Transport information

- Regulatory information

- Other information

- Labels

The GHS includes directions for application of the hazard communication elements on the label. In particular, it specifies for each hazard, and for each class within the hazard, what signal word, pictogram and hazard statement should be used.

A full list can be found in Appendix 3.

- Document Control

This document is maintained, controlled and formally reviewed by the University Safety Service. This document is maintained electronically and the latest issue will be displayed on the Safety Service Web Site. Printed copies shall be treated as uncontrolled documents.

Comments or queries relating to the contents of this document should be referred to:

University Safety Service, e-mail: safety@qub.ac.uk

| Document Reference | SS/GU/014/1 |
| --- | --- |
| Date Issued | April 2014 |
| Version | 01 |
| Next Review Date | April 2016 |
| Supersedes | None |
| Consultation undertaken | August 2014 |
| Supporting Procedure(s) | None |
| Document approved by: | Compliance Committee |
| Document Owner | Safety Service |
| Lead Contact | L Smith |

Contents

1.	Introduction									1

2.	Scope										1

3.	Pictograms									1

4.	Signal Words									3

5.	Hazard Statements								3

6.	Precautionary Statements							3

7.	Safety Data Sheets								4

8.	Labels										5

9.	Document Control								5

- Contents									6

Appendix 1 – GHS Hazard Statements					7

Appendix 2 – GHS Precautionary Statements				10

Appendix 3 – The new classification and labelling system according	14

to CLP regulation (EC) No. 1272/2008

Appendix 1 – GHS Hazard Statements

GHS Physical Hazard Statements

| Hazard Code | Hazard Statement |
| --- | --- |
| H200 | Unstable, explosive |
| H201 | Explosive, mass explosion hazard |
| H202 | Explosive, severe projection hazard |
| H203 | Explosive, fire, blast or projection hazard |
| H204 | Fire or projection hazard |
| H205 | May mass explode in fire |
| H220 | Extremely flammable gas |
| H221 | Flammable gas |
| H222 | Extremely flammable aerosol |
| H223 | Flammable aerosol |
| H224 | Extremely flammable liquid and vapour |
| H225 | Highly flammable liquid and vapour |
| H226 | Flammable liquid and vapour |
| H227 | Combustible liquid |
| H228 | Flammable solid |
| H240 | Heating may cause explosion |
| H241 | Heating may cause a fire or explosion |
| H242 | Heating may cause a fire |
| H250 | Catches fire spontaneously if exposed to air |
| H251 | Self heating, may catch fire |
| H252 | Self heating in large quantities, may catch fire |
| H260 | In contact with water releases flammable gases which may ignite spontaneously |
| H261 | In contact with water, releases flammable gas |
| H270 | May cause or intensify fire; oxidiser |
| H271 | May cause fire or explosion; strong oxidiser |
| H272 | May intensify fire; oxidiser |
| H280 | Contains gas under pressure; may explode if heated |
| H281 | Contains refrigerated gas; may cause cryogenic burns or injury |
| H290 | May be corrosive to metals |

GHS Health Hazard Statements

| Hazard Code | Hazard Statement |
| --- | --- |
| H300 | Fatal if swallowed |
| H301 | Toxic if swallowed |
| H302 | Harmful if swallowed |
| H303 | May be harmful if swallowed |
| H304 | May be fatal if swallowed and enters airways |
| H305 | May be harmful if swallowed and enters airways |
| H310 | Fatal in contact with skin |
| H311 | Toxic in contact with skin |
| H312 | Harmful in contact with skin |
| H313 | May be harmful in contact with skin |
| H314 | Causes severe skin burns and eye damage |
| H315 | Causes skin irritation |
| H316 | Causes mild skin irritation |
| H317 | May cause an allergic skin reaction |
| H318 | Causes serious eye damage |
| H319 | Causes serious eye irritation |
| H320 | Causes eye irritation |
| H330 | Fatal if inhaled |
| H331 | Toxic if inhaled |
| H332 | Harmful if inhaled |
| H333 | May be harmful if inhaled |
| H334 | May cause allergy or asthma symptoms or breathing difficulties if inhaled |
| H335 | May cause respiratory irritation |
| H336 | May cause drowsiness or dizziness |
| H340 | May cause genetic defects |
| H341 | Suspected of causing genetic defects |
| H350 | May cause cancer |
| H350I | May cause cancer by inhalation |
| H351 | Suspected of causing cancer |
| H360 | May damage fertility or the unborn child |
| H360F | May damage fertility |
| H360D | May damage the unborn child |
| H360FD | May damage fertility and the unborn child |
| H360Fd | May damage fertility; Suspected of damaging unborn child |
| H360Df | May damage the unborn child; Suspected of damaging fertility |
| H361 | Suspected of damaging fertility or the unborn child |
| H361f | Suspected of damaging fertility |
| H361d | Suspected of damaging the unborn child |
| H361fd | Suspected of damaging fertility; Suspected of damaging the unborn child |
| H362 | May cause harm to breast fed children |
| H370 | Causes damage to organs |
| H371 | May cause damage to organs |
| H372 | Causes damage to organs through prolonged or repeated exposure |
| H373 | May cause damage to organs through prolonged or repeated exposure |

GHS Environmental Hazard Statements

| Hazard Code | Hazard Statement |
| --- | --- |
| H400 | Very toxic to aquatic life |
| H401 | Toxic to aquatic life |
| H402 | Harmful to aquatic life |
| H410 | Very toxic to aquatic life with long lasting effects |
| H411 | Toxic to aquatic life with long lasting effects |
| H412 | Harmful to aquatic life with long lasting effects |
| H413 | May cause long lasting harmful effects to aquatic life |
| H420 | Harms public health and the environment by destroying ozone in the upper atmosphere |

Appendix 2 – GHS Precautionary Statements

GHS Precautionary Statements – General Actions

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P101 | If medical advice is needed, have product container or label to hand |
| P102 | Keep out of reach of children |
| P103 | Read label before use |

GHS Precautionary Statements – Preventative Measures

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P201 | Obtain special instructions before use |
| P202 | Do not handle until all safety precautions have been read and understood |
| P210 | Keep away from heat/sparks/open flames/hot surfaces – no smoking |
| P211 | Do not spray onto an open flame or other ignition source |
| P220 | Keep/store away from clothing/…/combustible materials |
| P221 | Take any precaution to avoid mixing with combustible materials |
| P222 | Do not allow contact with air |
| P223 | Keep away from any possible contact with water because of violent reaction and possible flash fire |
| P230 | Keep wetted with…. |
| P231 | Handle under inert gas |
| P232 | Protect from moisture |
| P233 | Keep container tightly closed |
| P234 | Keep only in original container |
| P235 | Keep cool |
| P240 | Ground/bond container and receiving equipment |
| P241 | Use explosion proof electrical/ventilation/lighting/…/equipment |
| P242 | Use only non sparking tools |
| P243 | Take precautionary measures against static discharge |
| P244 | Keep reduction valves free from grease and oil |
| P250 | Do not subject to grinding/shock/../friction |
| P251 | Pressurised container: do not pierce or burn, even after use |
| P260 | Do not breathe dust/fumes/gas/mist/vapours/spray |
| P261 | Avoid breathing dust/fumes/gas/mist/vapours/spray |
| P262 | Do not get in eyes, on skin or on clothing |
| P263 | Avoid contact during pregnancy/while nursing |
| P264 | Wash thoroughly after handling |
| P270 | Do not eat, drink or smoke when using this product |

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P271 | Use only outdoors or in a well ventilated area |
| P272 | Contaminated work clothing should not be allowed out of the workplace |
| P273 | Avoid release to the environment |
| P280 | Wear protective gloves/protective clothing/eye protection/face protection |
| P281 | Use personal protective equipment as required |
| P282 | Wear cold insulating gloves/face shield/eye protection |
| P283 | Wear fire/flame resistant/retardant clothing |
| P284 | Wear respiratory protection |
| P285 | In case of inadequate ventilation wear respiratory protection |
| P231 | Handle under inert gas. Protect from moisture |
| P232 | Handle under inert gas. Protect from moisture |
| P235 | Keep cool. Protect from sunlight |

GHS Precautionary Statements – Emergency Response

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P301 | If swallowed: |
| P302 | If on skin: |
| P303 | If on skin (or hair): |
| P304 | If inhaled: |
| P305 | If in eyes: |
| P306 | If on clothing: |
| P307 | If exposed: |
| P308 | If exposed or concerned: |
| P309 | If exposed or you feel unwell |
| P310 | Immediately call a poison centre or doctor/physician |
| P311 | Call a poison centre or doctor/physician |
| P312 | Call a poison centre or doctor/physician if you feel unwell |
| P313 | Get medical advice/attention |
| P314 | Get medical advice/attention if you feel unwell |
| P315 | Get immediate medical advice/attention |
| P320 | Specific treatment is urgent (see…. on this label) |
| P321 | Specific treatment (see…on this label) |
| P322 | Specific measures (see…on this label) |
| P330 | Rinse mouth |

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P331 | Do not induce vomiting |
| P332 | If skin irritation occurs: |
| P333 | If skin irritation or rash occurs: |
| P334 | Immerse in cool water/wrap in wet bandages |
| P335 | Brush off loose particles from skin |
| P336 | Thaw frosted parts with lukewarm water. Do not rub affected area |
| P337 | If eye irritation persists |
| P338 | Remove contact lenses, if present and easy to do. Continue rinsing |
| P340 | Remove victim to fresh air and keep at rest in a position comfortable for breathing |
| P341 | If breathing is difficult, remove victim to fresh air and keep at rest in a position comfortable for breathing |
| P342 | If experiencing respiratory symptoms: |
| P350 | Gently wash with plenty of soap and water |
| P351 | Rinse cautiously with water for several minutes |
| P352 | Wash with plenty of soap and water |
| P353 | Rinse with water/shower |
| P360 | Rinse immediately contaminated clothing and skin with plenty of water before removing clothing |
| P361 | Remove/take off immediately all contaminated clothing |
| P362 | Take off contaminated clothing and wash before reuse |
| P363 | Wash contaminated clothing before reuse |
| P370 | In case of fire: |
| P371 | In case of major fire and large quantities: |
| P372 | Explosion risk in case of fire |
| P373 | Do not fight fire when fire reaches explosives |
| P374 | Fight fire with normal precautions from a reasonable distance |
| P375 | Fight fire remotely due to risk of explosion |
| P376 | Stop leak if safe to do so |
| P377 | Leaking gas fire: Do not extinguish, unless leak can be stopped safely |
| P378 | Use ….. for extinction |
| P380 | Evacuate area |
| P381 | Eliminate all ignition sources if safe to do so |
| P390 | Absorb spillage to prevent material damage |
| P391 | Collect spillage |

GHS Precautionary Statements – Storage Measures

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P401 | Store… |
| P402 | Store in a dry place |
| P403 | Store in a well ventilated place |
| P404 | Store in a closed container |
| P405 | Store locked-up |
| P406 | Store in corrosive resistant/…/container with a resistant liner |
| P407 | Maintain air gap between pallets |
| P410 | Protect from sunlight |
| P411 | Store at temperatures not exceeding…..oC |
| P412 | Do not expose to temperatures exceeding 50oC |
| P413 | Store bulk masses greater than ….kg at temperatures not exceeding …. oC |
| P420 | Store away from other materials |
| P422 | Store contents under …… |

GHS Precautionary Statements – Disposal

| Precautionary Code | Precautionary Statement |
| --- | --- |
| P501 | Dispose of contents/container to……. |
| P502 | Refer to manufacturer/supplier for information on recovery/recycling |

Appendix 3 – The new classification and labelling system according to CLP regulation (EC) No. 1272/2008

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Explosives | Unstable Explosive |  | Danger | H200 | Unstable Explosive | P201, P202, P281, P372, P373, P380, P401, P501 |
|  | Div 1.1 |  | Danger | H201 | Explosive; mass Explosion hazard | P210, P230, P240, P250, P280, P370+P380, P372, P373, P401, P501 |
|  | Div 1.2 |  | Danger | H202 | Explosive; severe Projection hazard | P210, P230, P240, P250, P280, P370+P380, P372, P373, P401, P501 |
|  | Div 1.3 |  | Danger | H203 | Explosive; fire, blast or projection hazard | P210, P230, P240, P250, P280, P370+P380, P372, P373, P401, P501 |
|  | Div 1.4 |  | Warning | H204 | Fire or projection hazard | P210, P240, P250, P280, P370+P380, P372, P373, P374, P401, P501 |
|  | Div 1.5 | No pictogram | Danger | H205 | May mass explode in fire | P210, P230, P234, P240, P250, P280, P370+P380, P372, P373, P401, P501 |
|  | Div 1.6 | No pictogram |  |  |  |  |
| Flammable Gases | Category1 |  | Danger | H220 | Extremely flammable gas | P210, P377, P381, P403 |
|  | Category 2 | No pictogram | Warning | H221 | Flammable gas | P210, P377, P381, P403 |
| Flammable Aerosols | Category 1 |  | Danger | H222 | Extremely flammable aerosol | P210, P211, P251, P410+P412 |
|  | Category 2 |  | Warning | H223 | Flammable aerosol | P210, P211, P251, P410+P412 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Flammable Liquids | Category 1 |  | Danger | H224 | Extremely flammable liquid and vapour | P210, P233, P240, P241, P242, P243, P280, P303+P361+P353, P370+P378, P403+P235, P501 |
|  | Category 2 |  | Danger | H225 | Highly flammable liquid and vapour |  |
|  | Category 3 |  | Warning | H226 | Flammable liquid and vapour |  |
| Flammable Solids | Category 1 |  | Danger | H228 | Flammable Solid | P210, P240, P241, P280, P370+P378 |
|  | Category 2 |  | Warning | H228 | Flammable Solid |  |
| Self-reactive substances and mixtures; and organic peroxides | Type A |  | Danger | H240 | Heating may cause an explosion | P210, P220, P234, P280, P370+P378, P370+P380+P375, P403+P235, P411, P420, P501 |
|  | Type B |  | Danger | H241 | Heating may cause a fire or explosion | P210, P220, P234, P280, P370+P378, P370+P380+P375, P403+P235, P411, P420, P501 |
|  | Type C, D |  | Danger | H242 | Heating may cause a fire | P210, P220, P234, P280, P370+P378, P403+P235, P411, P420, P501 |
|  | Type E, F |  | Warning | H242 | Heating may cause a fire | P210, P220, P234, P280, P370+P378, P403+P235, P411, P420, P501 |
|  | Type G | No pictogram |  |  |  |  |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Pyrophoric liquids; pyrophoric solids | Category 1 |  | Danger | H250 | Catches fire spontaneously if exposed to air | P210, P222, P231, P280, P302+P334, P370+P378, P422 |
| Self-heating substances and mixtures | Category 1 |  | Danger | H251 | Self-heating; may catch fire | P235+P410, P280, P407, P413, P420 |
|  | Category 2 |  | Warning | H252 | Self-heating in large quantities; may catch fire |  |
| Substances and mixtures which, in contact with water, emit flammable gases | Category 1 |  | Danger | H260 | In contact with water releases flammable gases which may ignite spontaneously | P223, P231+P232, P280, P335+P334, P370+P378, P402+P404, P501 |
|  | Category 2 |  | Danger | H261 | In contact with water releases flammable gas |  |
|  | Category 3 |  | Warning | H261 | In contact with water releases flammable gas | P231+P232, P280, P370+P378, P402+P404, P501 |
| Oxidising gases | Category 1 |  | Danger | H270 | May cause or intensify fire; oxidiser | P220, P244, P370+P376, P403 |
| Oxidising liquids; Oxidising solids | Category 1 |  | Danger | H271 | May cause fire or explosion; strong oxidiser | P210, P220, P221, P280, P283, P306+P360, P371+P380+P375, P370+P378, P501 |
|  | Category 2 |  | Danger | H272 | May intensify fire; oxidiser | P210, P220, P221, P280, P370+P378, P501 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Gases under pressure | Compressed gas Liquefied gas Dissolved gas |  | Warning | H280 | Contains gas under pressure; may explode if heated | P410+P403 |
|  | Refrigerated liquefied gas |  | Warning | H281 | Contains refrigerated gas; may cause cryogenic burns or injury | P282, P336, P315, P403 |
| Corrosive to metals | Category 1 |  | Warning | H290 | May be corrosive to metals | P234, P390, P406 |
| Acute toxicity, oral | Category 1, 2 |  | Danger | H300 | Fatal if swallowed | P264, P270, P301+P310, P321, P330, P405, P501 |
|  | Category 3 |  | Danger | H301 | Toxic if swallowed |  |
|  | Category 4 |  | Warning | H302 | Harmful if swallowed | P264, P270, P301+P312, P330, P501 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Aspiration hazard | Category 1 |  | Danger | H304 | May be fatal if swallowed and enters airways | P301+P310, P331, P405, P501 |
| Acute toxicity, dermal | Category 1, 2 |  | Danger | H310 | Fatal in contact with skin | P262, P264, P270, P280, P302+P350, P310, P322, P361, P363, P405, P501 |
|  | Category 3 |  | Danger | H311 | Toxic in contact with skin | P280, P302+P352, P312, P322, P361, P363, P405, P501 |
|  | Category 4 |  | Warning | H312 | Harmful in contact with skin | P280, P302+P35P312, P322, P363, P501 |
| Skin corrosion / irritation | Category 1A, 1B, 1C |  | Danger | H314 | Causes severe skin burns and eye damage | P260, P264, P280, P301+P330+P331, P303+P364+P353, P363, P304+P340, P310, P321, P305+P351+P338, P405, P501 |
|  | Category 2 |  | Warning | H315 | Causes skin irritation | P264, P280, P302+P352, P321, P332+P313, P362 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Sensitisation, Skin | Category 1, Sub-category 1A Sub-category 1B |  | Warning | H317 | May cause an allergic skin reaction | P261, P272, P280, P302+P352, P333+P313, P321, P363, P501 |
| Serious eye damage / eye irritation | Category 1 |  | Danger | H318 | Causes serious eye damage | P280, P305+P351+P338, P310 |
|  | Category 2 |  | Warning | H319 | Causes serious eye irritation | P264, P280, P305+P351+P338, P337+P313 |
| Acute toxicity, inhalation | Category 1, 2 |  | Danger | H330 | Fatal if inhaled | P260, P271, P284, P304+P340, P310, P320, P403+P233, P405, P501 |
|  | Category 3 |  | Danger | H331 | Toxic if inhaled | P261, P271, P304+P340, P311, P321, P403+P233, P405, P501 |
|  | Category 4 |  | Warning | H332 | Harmful if inhaled | P261, P271, P304+P340, P312 |
| Sensitisation, respiratory | Category 1, Sub-category 1A Sub-category 1B |  | Danger | H334 | May cause allergy or asthma symptoms or breathing difficulties if inhaled | P261, P285, P304+P341, P342+P311, P501 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Specific target organ toxicity, single exposure; Respiratory tract irritation | Category 3 |  | Warning | H335 | May cause respiratory irritation | P261, P271, P304+P340, P312, P403+P233, P405, P501 |
| Specific target organ toxicity, single exposure; narcotic effects | Category 3 |  | Warning | H336 | May cause drowsiness or dizziness |  |
| Germ cell mutagenicity | Category 1A, 1B |  | Danger | H340 | May cause genetic defects | P201, P202, P281, P308+P313, P405, P501 |
|  | Category 2 |  | Warning | H341 | Suspected of causing genetic defects |  |
| Carcinogenicity | Category 1A, 1B |  | Danger | H350 | May cause cancer | P201, P202, P281, P308+P313, P405, P501 |
|  |  |  |  | H350i | May cause cancer if inhaled |  |
|  | Category 2 |  | Warning | H351 | Suspected of causing cancer |  |
| Reproductive toxicity | Category 1A, 1B |  | Danger | H360 | May damage fertility or the unborn child | P201, P202, P281, P308+P313, P405, P501 |
|  |  |  |  | H360F | May damage fertility |  |
|  |  |  |  | H360D | May damage the unborn child |  |
|  |  |  |  | H360FD | May damage fertility; may damage the unborn child |  |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Reproductive toxicity | Category 2 |  | Warning | H361 | Suspected of damaging fertility or the unborn child | P201, P202, P281, P308+P313, P405, P501 |
|  |  |  |  | H361f | Suspected of damaging fertility |  |
|  |  |  |  | H361d | Suspected of damaging the unborn child |  |
|  |  |  |  | H361fd | Suspected of damaging fertility; suspected of damaging the unborn child |  |
| Reproductive toxicity, effects on or via lactation | Additional category | No pictogram |  | H362 | May cause harm to breast-fed children | P201, P260, P263, P264, P270, P308+P313 |
| Specific target organ toxicity, single exposure | Category 1 |  | Danger | H370 | Causes damage to organs | P260, P264, P270, P307+P311, P321, P405, P501 |
|  | Category 2 |  | Warning | H371 | May cause damage to organs | P260, P264, P270, P309+P311, P405, P501 |
| Specific target organ toxicity, repeated exposure | Category 1 |  | Danger | H372 | Causes damage to organs through prolonged or repeated exposure | P260, P264, P270, P314, P501 |
|  | Category 2 |  | Warning | H373 | Causes damage to organs through prolonged or repeated exposure | P260, P314, P501 |

| Hazard Class | Category | Pictogram | Signal Word | Code | Hazard Statements | P-Codes |
| --- | --- | --- | --- | --- | --- | --- |
| Hazardous to the aquatic environment, acute hazard | Category 1 |  | Warning | H400 | Very toxic to aquatic life | P273, P391, P501 |
| Hazardous to the aquatic environment, chronic hazard | Category 1 |  | Warning | H410 | Very toxic to aquatic life with long lasting effects | P273, P391, P501 |
|  | Category 2 |  |  | H411 | Toxic to aquatic life with long lasting effects |  |
|  | Category 3 | No pictogram |  | H412 | Harmful to aquatic life with long lasting effects | P273, P501 |
|  | Category 4 | No pictogram |  | H413 | May cause long lasting harmful effects to aquatic life |  |
| Hazardous to the ozone layer | Ozone 1 |  | Warning | H420 | Harms public health and the environment by destroying ozone in the upper atmosphere | P502 |